Technical assignment — Paid research stay selection

Dear candidate,

As part of the selection procedure for a paid research stay at the Rehabilitation Research group, VUB, we invite you to complete a short technical assignment.

You will receive:

- one CSV file of a novice Tai Chi execution;

- one CSV file of a trained Tai Chi execution;

- one MP4 video of the novice execution;

- one figure showing the IMU positions.

The aim is not to produce a full final analysis, but to assess how you reason from raw IMU signals toward balance-relevant movement variability.

The central question is:

Which IMU-based differences between the novice and trained Tai Chi execution may provide useful insight into balance control?

Tai Chi consists of slow, controlled movements involving weight shifting, trunk and pelvis control, rotations, postural transitions, and moments of quasi-static balance. Therefore, please do not treat the full Tai Chi recording as one repeated cyclic movement. We are especially interested in how you identify meaningful movement modules or events that are relevant for balance control.

Examples of such modules may include weight shifts, postural transitions, trunk or pelvis rotations, start-stop phases, or quasi-static balance moments. These examples are not exhaustive; other justified choices are welcome.

Assignment

Please submit a short document of maximum 2 pages, including one figure.

Address the following points:

- Select one or two balance-relevant events

Use the novice video to identify one or two movement events that are relevant for balance control.

Briefly explain:

- what the event is;

- where it occurs in the novice recording;

- why it is relevant for balance.

- Segmentation or alignment of novice and trained recordings

Explain how you would identify the corresponding event or a comparable signal pattern in the trained CSV, where no video is available.

The key point is to avoid a purely global comparison of the full recordings without considering the structure of the movement.

- Select 2–3 IMU-based variability outputs

Propose 2–3 outputs that could compare novice and trained performance in relation to balance.

Examples may include:

- medio-lateral trunk or pelvis acceleration variability;

- trunk or pelvis angular velocity peaks;

- smoothness or jerk of trunk/pelvis movement;

- duration or variability of postural transitions;

- number of small corrective peaks around quasi-static phases;

- trunk-pelvis coordination or timing;

- left-right asymmetry if relevant.

For each output, explain briefly:

- what it measures;

- why it is relevant for balance;

- how a novice-to-trained change could be interpreted.

Please avoid assuming that less variability is always better. Lower variability may reflect better control, but it may also reflect stiffness or overcontrol. Higher variability may reflect instability, but it may also reflect functional flexibility.

4. Provide one traceability figure

Include one figure that shows your reasoning from data to interpretation.

The figure should include:

- a raw or minimally processed IMU signal;

- the selected event/module boundaries;

- a novice-versus-trained comparison where feasible;

- one balance-relevant marker or visual indication of the difference.

The goal of this figure is to show that your interpretation is grounded in the actual dataset.

4. Provide one dataset-specific observation

Include one concrete observation that could only come from inspecting the files.

For example:

- which IMU sensor was most informative;

- which signal was noisy or difficult to interpret;

- which visible movement in the novice video matched a clear IMU pattern;

- whether the trained execution appeared shorter, smoother, more abrupt, or differently structured;

- which comparison was least reliable.

5. Brief limitations and feasibility

End with a short paragraph on:

- the main limitations of this mini-analysis;

- what could realistically be developed further during a 2-month research stay.

Please mention whether AI tools were used, and if so, for what purpose. Use of AI for language editing, coding support, or debugging is allowed, but the interpretation, figures, and methodological choices must be your own.

Expected output

Please submit:

- a short technical note, maximum 2 pages;

- maximum 1 additional page with figure(s);

- optional code or pseudocode, if useful;

- a short statement on which parts of the proposed workflow would be feasible during a 2-month research stay.

The strongest answer will not necessarily be the most complex analysis. We are looking for someone who can reason clearly from raw multi-IMU signals toward a reproducible and biomechanically meaningful interpretation of movement variability in relation to balance control.